# NCS to RGB Hue Mapping: Problem and Solution

## Problem Description

We are given hue values from the Natural Color System (NCS), which distributes hues evenly around a color wheel with four primary reference points:

- Yellow (Y) at 0°
- Red (R) at 90°
- Blue (B) at 180°
- Green (G) at 270°

The NCS color circle is thus divided into four equal 90° segments, each representing a primary hue quadrant:

1. Yellow to Red: 0° to 90°
2. Red to Blue: 90° to 180°
3. Blue to Green: 180° to 270°
4. Green to Yellow: 270° to 360°

The problem asks to map these NCS degrees to another hue system (the RGB hue wheel) that has a different configuration and non-linear spacing between its primary hues. The RGB hue wheel places its four primary hues at:

- Red at 0° (or equivalently 360°)
- Yellow at 60°
- Green at 120°
- Blue at 240°

Unlike NCS, the RGB intervals are not uniform. The shortest paths are:

- Red → Yellow = 60°
- Yellow → Green = 60°
- Green → Blue = 120°
- Blue → Red = 120° (shortest when decreasing from 360° to 240°)

The Goal: To create a function or mapping logic that, given any NCS hue degree, returns the corresponding RGB hue degree. This mapping must:

1. Identify the NCS quadrant.
2. Compute a proportional mapping based on the position within that quadrant.
3. Translate that proportion into the corresponding RGB hue range.
4. Always choose the shortest path on the RGB hue wheel.
5. Handle a special case for hues in the Red quadrant (90° < NCS < 180°) due to the directional change required on the RGB wheel.

## Key Concepts

NCS Quadrants:
The NCS hue circle is divided into four equal quadrants of 90° each:

- Yellow (Y): 0°
- Red (R): 90°
- Blue (B): 180°
- Green (G): 270°

RGB Hue Wheel:
The RGB hue wheel places primary hues at different intervals:

- Red: 0° (or 360°)
- Yellow: 60°
- Green: 120°
- Blue: 240°

Mapping Approach:
1. Identify the NCS quadrant.
2. Calculate the proportion of the NCS degree within the quadrant.
3. Map the proportion to the corresponding RGB span.
4. Handle the special case for the Red quadrant.
5. Normalize the output within [0°, 360°).

## Detailed Example

Example: NCS = 150°

1. Quadrant: Red quadrant since 90° ≤ 150° < 180°.
2. Proportion:
   p = (150 - 90) / (180 - 90) = 60 / 90 = 0.6667
3. Special Case Mapping for Red Quadrant:
   RGB Output = 360 - (p × 120) = 360 - (0.6667 × 120) = 280°

Thus, NCS 150° maps to RGB 280°.

## Verification

The logic has been verified against multiple test inputs spanning all quadrants, including boundary values and internal values. Each tested input produced the expected output, confirming that:

- The proportions are correctly calculated.
- The special case for the Red quadrant is correctly handled.
- The shortest path rule on the RGB hue wheel is maintained.

## Conclusion

By following the outlined steps:

1. Identify the quadrant on the NCS circle.
2. Calculate the proportion within that quadrant.
3. Apply the correct RGB mapping logic, using special case handling for the Red quadrant.
4. Ensure direction is chosen to always follow the shortest path.
5. Normalize the final angle.

This solution robustly converts any given NCS degree to the corresponding RGB degree, accurately reflecting both the non-uniform intervals on the RGB hue wheel and the need for shortest-path traveling between colors.